XDS Sigma 32b Simulator Usage

27-Mar-2016

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2016, written by Robert M Supnik

Copyright (c) 1993-2016, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	XDS Sigma 32b Features	3

2.1	Central Processor (CPU)	4

2.2	Memory Map (MAP)	5

2.3	Interrupt System (INT)	6

2.4	Channels (CHANA .. CHANH)	6

2.5	7012 Console Teletype (TT)	7

2.6	7060 Paper Tape Reader (PT)	8

2.7	7440 or 7450 Line Printer (LP)	8

2.8	Real-Time Clock (RTC)	9

2.9	Character-Oriented Communications Subsystem (MUX)	10

2.10	7240, 7270, 7260, 7265, 7275, and T3281 Moving Head Disk Controllers (DPA, DPB)	11

2.11	7250 Cartridge Disk Controller (DK)	12

2.12	7211/7212 or 7231/7232 Fixed Head Disk (RAD)	13

2.13	723X Nine-Track Magnetic Tape (MT)	13

3	Symbolic Display and Input	14

This memorandum documents the SDS 940 simulator.

# Simulator Files

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/sds/	sigma_defs.h

sigma_io_defs.h

sigma_cis.c

sigma_coc.c

sigma_cpu.c

sigma_dk.c

sigma_dp.c

sigma_fp.c

sigma_io.c

sigma_lp.c

sigma_map.c

sigma_mt.c

sigma_pt.c

sigma_rad.c

sigma_rtc.c

sigma_sys.c

sigma_tt.c

# XDS Sigma 32b Features

The XDS Sigma 32b simulator is configured as follows:

device name(s)	simulates

CPU			XDS Sigma 5, 6, 7, 7 “big mem”, 8, 9, or XDS 550, 560

CHANA..CHANH	I/O channels, up to 8

PT			7060 paper tape reader/punch

TT			7012 console Teletype

LP			7440 or 7450 line printer

RTC			real-time clock

MUX			character-oriented communications subsystem

DK			7250 cartridge disk subsystem with up to 8 drives

DPA			disk pack subsystem with up to 15 drives

DPB			disk pack subsystem with up to 15 drives

RAD			7211/7212 or 7231/7232 fixed head disk

MT			732X 9-track magnetic tape with up to 8 drives

Most devices can be disabled or enabled with the SET <dev> DISABLED and SET <dev> ENABLED commands, respectively.

The LOAD and DUMP commands are not implemented.

## Central Processor (CPU)

Central processor options include the CPU model, the CPU features, and the size of main memory.

SET CPU SIGMA5			set CPU model to Sigma 5

SET CPU SIGMA6			set CPU model to Sigma 6

SET CPU SIGMA7			set CPU model to Sigma 7

SET CPU SIGMA7B			set CPU model to Sigma 7 “big” memory

SET CPU SIGMA8			set CPU Model to Sigma 8

SET CPU SIGMA9			set CPU model to Sigma 9

SET CPU 550				set CPU model to 550

SET CPU 560				set CPU model to 560

SET CPU FP				enable floating point, if available

SET CPU NOFP			disable floating point, if optional

SET CPU DECIMAL			enable decimal, if available

SET CPU NODECIMAL			disable decimal, if optional

SET CPU LASLAM			enable LAS/LAM instructions (6/7 only)

SET CPU NOLASLAM			disable LAS/LAM instructions

SET CPU MAP				enable memory map, if available

SET CPU NOMAP			disable memory map, if optional

SET CPU WRITELOCK			enable write locks, if available

SET CPU NOWRITELOCK		disable write locks, if optional

SET CPU RBLKS=n			set number of register blocks

SET CPU CHAN=n			set number of channels

SET CPU 32K				set memory size = 32KW

SET CPU 64K				set memory size = 64KW

SET CPU 128K			set memory size = 64KW

SET CPU 256K			set memory size = 256KW

SET CPU 512K			set memory size = 512KW

SET CPU 1M				set memory size = 1024KW

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial configuration is Sigma 7 CPU, 4 channels, 128KW of memory, floating point, decimal, map and writelocks options enabled.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			17		program counter

R0..R15		32		active general registers

PSW1			32		processor status word 1

PSW2			32		processor status word 2

PSW4			32		processor status word 4 (5x0 only)

CC			4		condition codes

RP			5		register block selector

SSW1..SSW4		1		sense switches

PDF			1		processor fault flag

ALARM			1		console alarm

ALENB			1		console alarm enable

PCF			1		console controlled flop

EXULIM		8		limit for nested EXU’s

STOP_ILL		1		if 1, stop on undefined instruction

REG			512		register blocks, 32 x 16

WRU			8		interrupt character

The CPU provides an address converter to display the byte (halfword, word, or doubleword) address equivalent of a byte (halfword, word, or doubleword) input address. Optionally, the input address can be run through memory relocation:

SHOW {-flags} CPU {BA,HA,WA,DA}=address

BA, HA, WA, DA specify that the input address is a byte, halfword, word, or doubleword address, respectively.  The flags are:

-v			input address is virtual

-b			output is byte address

-h			output is halfword address

-w			output is word address (default)

-d			output is doubleword address

For example:

SHOW –B CPU WA=100

Physical word 100: physical byte 400

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 1M entries.

## Memory Map (MAP)

The memory map implements two distinct forms of protection: memory mapping and access protection on virtual addresses; and write lock protection on physical addresses.  It also includes a skeleton implementation of the memory status registers from the Sigma 8 and 9, and the XDS 550 and 560.  It implements these registers:

name			size		comments

REL[0:511]		11		relocation registers

ACC[0:511]		2		access controls

WLK[0:2047]		4		write locks

SR0[32]		32		memory status register 0

SR1[32]		32		memory status register 1

## Interrupt System (INT)

The Sigma series implements a complex, multi-level interrupts system, with a minimum of 32 distinct interrupts. The interrupt system can be expanded to up to 224 interrupt levels. It implements these registers:

name			size		comments

IHIACT		9		highest active interrupt level

IHIREQ		9		highest outstanding interrupt request

IREQ[0:15]		16		interrupt requests

IENB[0:15]		16		interrupt enabled flags

IARM[0:15]		16		interrupt armed flags

S9_SNAP		32		Sigma 9 snapshot register

S9_MARG		32		Sigma 9 margins registers

S5X0_IREG[0:31]	32		5X0 internal registers (unused)

The simulator supports a variable number of external interrupt blocks. The minimum number is one, the maximum is four (5X0) to thirty-two (Sigma 7). The user can change the number of external interrupt blocks with the SET INT EIBLKS command:

SET INT EIBLKS=4			configure four external interrupt blocks

Although the Sigma series supports configurable interrupt group priorities, the simulator uses a fixed priority arrangement, high to low as follows:

- counters

- counter overflow

- I/O and panel interrupt

- external group 2 (there is no external group 1)

- external group 3

- etc.

## Channels (CHANA .. CHANH)

A Sigma 32b system has up to eight I/O channels, designated A, B, C, D, E, F, G, and H.  The association between a device and a channel is displayed by the SHOW <dev> CHAN command and SHOW <dev> DVA command:

SHOW MT CHAN

channel=A

SHOW MT DVA

address=00

The user can change the association with the SET <dev> CHAN=<chan> command, where <chan> is a channel letter, and with the SET <dev> DVA=<addr> command, where <addr> is a legal device address:

SET MT CHAN=C

SET MT DVA=4

SHOW MT CHAN,DVA

channel=B

address=04

The default channel assignments for the simulator are:

Device	Channel		Address

MT		chan A		address 0

TT		chan A		address 1

LP		chan A		address 2

PT		chan A		address 5

COC		chan A		address 6

RAD		chan B		address 0

DK		chan B		address 1

DPA		chan C		address 0

DPB		chan D		address 1

The user can display all the channel registers associated with a device with the SHOW <dev> CSTATE command:

SHOW MT CSTATE

Each channel has eight registers.  The registers are arrays, with entry 1 for device [0], entry [1] for device 1, etc.  A channel supports a maximum of 32 devices.

name			size		comments

CLC[0:31]		16		channel location counter (doubleword)

CMD[0:31]		8		current channel command

CMF[0:31]		8		channel command flags

BA[0:31]		24		byte address (byte)

BC[0:31]		16		byte count

CMF[0:31]		16		channel status flags

CHI[0:31]		8		channel interrupt flags

CHSF[0:31]		8		channel simulator flags

## 7012 Console Teletype (TT)

The console Teletype (TT) consists of two units. Unit 0 is for console input, unit 1 for console output. The console implements these registers:

name			size		comments

KPOS			32		number of characters input

TPOS			32		number of characters input

TTIME			24		time from I/O initiation to interrupt

PANEL			8		character code for panel interrupt

The PANEL variable defaults to control-P. If the user types the control panel character, it is not echoed; instead, a control panel interrupt is generated.

The console can be set to one of two modes, 7P or UC:

mode	input characters		output characters

UC	lower case converted	lower case converted

to upper case		to upper case

7P					non-printing characters suppressed

The default mode is UC. The console character set is a subset of EBCDIC.  By default, the console is assigned to channel A as device 1.

## 7060 Paper Tape Reader (PT)

The paper tape controller implements two units. Unit 0 (PT0) is for paper tape input, unit 1 (PT1) for paper tape output.  Paper tapes are simulated as disk files.  For the reader, register RPOS specifies the number of the next data item to be read.  For the punch, register PPOS specifies the number of the next data item to be written.  Thus, by changing RPOS or PPOS, the user can backspace or advance the reader or punch.

The paper tape controller implements these registers:

name			size		comments

CMD			9		current command or state

NZC			1		non-zero character seen since ATTACH

RPOS			32		position in the reader input file

RTIME			24		time from reader I/O initiation to response

RSTOP_IOE		1		stop on reader I/O error

PPOS			32		position in the punch output file

PTIME			24		time from punch I/O initiation to response

PSTOP_IOE		1		punch stop on I/O error

The paper-tape reader supports the BOOT command.  BOOT PT0 simulates the standard console fill sequence.

Reader error handling is as follows:

error	        RSTOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

Punch error handling is as follows:

error	        PSTOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

By default, the paper tape reader is assigned to channel A as device 5.

## 7440 or 7450 Line Printer (LP)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements these registers:

name			size		comments

CMD			9		current command or state

BUF[0:131]		7		data buffer

PASS			1		printing pass (7440 only)

INH			1		inhibit spacing flag

RUNAWAY		1		runaway carriage-control tape flag

CCT[0:131]		8		carriage control tape

CCTP			8		pointer into carriage control tape

CCTL			8		length of carriage control tape

POS			32		position in the output file

TIME			24		time from I/O initiation to response

STOP_IOE		1		stop on I/O error

The line printer model can be set to either the 7440 or the 7450:

SET LP 7440				set model to 7440

SET LP 7450				set model to 7450

The default model is the 7450.

A carriage control tape can be loaded with the SET LP CCT command:

SET LP CCT=<file>			load carriage-control tape from <file>

The format of a carriage control tape consists of multiple lines.  Each line contains an optional repeat count, enclosed in parentheses, optionally followed by a series of column numbers separated by commas.  Column numbers must be between 0 and 7.  The following are all legal carriage control specifications:

<blank line>			no punch

(5)					5 lines with no punches

1,5,7					columns 1, 5, 78 punched

(10)2					10 lines with column 2 punched

1					column 1 punched

The default form is 1 line long, with every column punched.

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

By default, the line printer is assigned to channel A as device 2.

## Real-Time Clock (RTC)

The Sigma 32b series implements four real-time clocks.  Three of them can be set to a variety of frequencies, including off, 50Hz, 60Hz, 100Hz, and 500hz. The fourth always runs at 500Hz.  The frequency of each clock can be can be adjusted as follows:

SET RTC {C1,C2,C3}=freq		set clock 1/2/3 to specified frequency

The frequency of each clock can be display as follows:

SHOW RTC {C1,C2,C3,C4}		show clock 1/2/3/4 frequency

Clocks 1 and 2 default to off, clocks 3 and 4 to 500Mhz.

The RTC can also show the state of all real-time events in the simulator:

SHOW RTC EVENTS

The real-time clocks autocalibrate; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.

## Character-Oriented Communications Subsystem (MUX)

The character-oriented communications subsystem implements up to 64 asynchronous interfaces, with modem control.  The subsystem has two controllers: MUX for the scanner, and MUXL for the individual lines.  The terminal multiplexer performs input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH MUX <port>			set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.

Unlike the console, the MUX operates in ASCII. Each line (each unit of MUXL) supports four character processing modes: UC, 7P, 7B, and 8B.

mode	input characters		output characters

UC	high-order bit cleared,	high-order bit cleared,

lower case converted	lower case converted

to upper case		to upper case

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default is UC.  In addition, each line supports output logging.  The SET MUXLn LOG command enables logging on a line:

SET MUXLn filename		log output of line n to filename

The SET MUXLn NOLOG command disables logging and closes the open log file, if any.

Once MUX is attached and the simulator is running, the multiplexer listens for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connections remain open until

disconnected either by the Telnet client, a SET MUX DISCONNECT command, or a DETACH MUX command.

Other special multiplexer commands:

SHOW MUX CONNECTIONS 		show current connections

SHOW MUX STATISTICS 		show statistics for active connections

SET MUXLn DISCONNECT		disconnects the specified line.

The controller (MUX) implements these registers:

name			size		comments

STA[0:63]		8		status, lines 0 to 63

RBUF[0:63]		8		receive buffer, lines 0 to 63

XBUF[0:63]		8		transmit buffer, lines 0 to 63

SCAN			6		current scanner line

SLCK			1		scanner locked flag

CMD			2		channel command or state

The lines (MUXL) implements these registers:

name			size		comments

TIME[0:63]		24		transmit time, lines 0 to 31

The terminal multiplexer does not support save and restore.  All open connections are lost when the simulator shuts down or MUX is detached.  By default, the multiplexer is assigned to channel A as device 6.

## 7240, 7270, 7260, 7265, 7275, and T3281 Moving Head Disk Controllers (DPA, DPB)

The Sigma 32b series supports two moving head disk controllers (DPA, DPB). Each can be set to model one of six controllers (7240, 7270, 7260, 7265, 7275, or Telefile 3281):

SET DP{A,B} 7240			set DPA (or DPB) to 7240

SET DP{A,B} 7270			set DPA (or DPB) to 7270

SET DP{A,B} 7260			set DPA (or DPB) to 7260

SET DP{A,B} 7265			set DPA (or DPB) to 7265

SET DP{A,B} 7275			set DPA (or DPB) to 7275

SET DP{A,B} T3281			set DPA (or DPB) to Telefile 3281

The default for DPA is the 7270, for DPB, the 3T281.

The 7240 and 7270 support up to 8 7242 and 7271 drives, respectively. The 7260, 7265, and 7275 support up to 15 7261, 7266, and 7276 drives, respectively. The T3281 supports up to 15 drives of three different types, which can be mixed:

Model     Cylinders         Heads          Sectors

3288         17              5               823

3282		 11		     19		   815

3283         17              19              815

The command to set a drive to a particular model is:

SET DPn <drive_type>		SET unit n to the specified drive type

The T3281 can also be set to autosize on ATTACH:

SET DPn AUTO			SET unit n to autosize on ATTACH

Units can be set ENABLED or DISABLED.  The DP controller supports the BOOT command.

The DP controllers implements the registers listed below.  Registers suffixed with [0:14] are replicated per drive.

name			size		comments

FLAGS			8		controller flags

DIFF			16		cylinder difference

SKI			16		queued seek interrupts

TEST			16		test mode flags

ADDR[0:14]		32		current disk address, drives 0 to 14

CMD[0:14]		32		current disk command, drives 0 to 14

TIME			24		time between word transfers

STIME			24		seek time

WLK			16		write lock switches

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

By default, DPA is assigned to channel C as device 0, and DPB is assigned to channel D as device 0.

## 7250 Cartridge Disk Controller (DK)

The cartridge disk controller (DK) supports up to 8 drives. DK options include the ability to make drives write enabled or write locked:

SET DKn LOCKED			set drive n write locked

SET DKn WRITEENABLED		set drive n write enabled

The cartridge disk controller implements these registers:

name			size		comments

CMD			9		current command or state

FLAGS			8		controller flags

ADDR			8		current disk address

TIME			24		interval between data transfers

STIME			24		seek interval

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		non-existent disk

DK data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur. By default, the cartridge disk controller is assigned to channel B as device 1.

## 7211/7212 or 7231/7232 Fixed Head Disk (RAD)

The Sigma 32b series supports two models of fixed head disk:

7211/7212		1.343MW

7231/7232		1.572MW

The user can select the model as follows:

SET RAD 7211			set model to 7211/7212

SET RAD 7231			set model to 7231/7232

The fixed head disk controller supports four units (drives). Units can be set ENABLED or DISABLED.

The fixed head disk controller implements these registers:

CMD			9		current command or state

FLAGS			8		controller flags

ADDR			15		current disk address

WLK			16		write lock switches

TIME			24		interval between data transfers

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		non-existent disk

RAD data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.  By default, the fixed head disk is assigned to channel B as device 0.

## 723X Nine-Track Magnetic Tape (MT)

The magnetic tape controller supports up to eight units.  MT options include the ability to make units write enabled or write locked.

SET MTn LOCKED			set unit n write locked

SET MTn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET MTn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW MTn CAPAC			show unit n capacity in MB

Units can also be set ENABLED or DISABLED.  The magnetic tape controller supports the BOOT command.  BOOT MTn simulates the standard console fill sequence for unit n.

The magnetic tape implements these registers:

name			size		comments

BUF[0:65535]	8		transfer buffer

BPTR			17		buffer pointer

BLNT			17		buffer length

RWINT			8		queued rewind interrupts

TIME			24		interval between character transfers

CTIME			24		channel response time

RWTIME		24		rewind time

UST[0:7]		8		drive status, drives 0 to 7

UCMD[0:7]		8		channel command, drives 0 to 7

POS[0:7]		32		position, drives 0 to 7

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		end of tape

OS I/O error	end of tape; if STOP_IOE, stop

By default, the magnetic tape is assigned to channel A as device 0.

# Symbolic Display and Input

The Sigma 32b simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as ASCII character (byte addressing)

-b			display as byte (byte addressing)

-e			display at EBCDIC character (byte addressing)

-h			display as halfword (halfword addressing)

-ca			display as four ASCII characters (word addressing)

-ce			display as four EBCDIC characters (word addressing)

-m			display instruction mnemonics (word addressing)

Input parsing is controlled by the first character typed in or by command line switches:

# or -a		ASCII character (byte addressing)

’ or –e		EBCDIC character (byte addressing)

-b			hexadecimal byte (byte addressing)

-h			hexadecimal halfword (halfword addressing)

" or -ac		four packed ASCII characters (word addressing)

-ae			four packed EBCDIC characters (word addressing)

alphabetic		instruction mnemonic (word addressing)

numeric		hexadecimal word (word addressing)

Instruction input uses (more or less) standard XDS Sigma assembler syntax. All instructions are variants on the same basic form:

mnemonic{,reg}	{*{address{,index}}}

Mnemonics are symbolic names for instructions. Registers are decimal values between 0 and 15. ‘*’ represents indirect addressing. Addresses are hexadecimal and can be signed if used as literals. Index registers are always less than 8 and thus can be considered decimal.

Examples:

LW,5		*100,7

AI,14		-3

LCFI		A

WAIT

BCR,12	400

BNOV		10FE

SLS,6		-8

The extended branch and shift mnemonics are recognized and decoded properly.